- Specifications BMW X5 M.

|  |  |  |  |  |  | BMW X5 M |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No of doors / seats |  |  |  |  |  | 5 / 5 |  |  |  |  |  |  |  |  |
| Length/width/height (unladen) | mm |  |  |  |  | 4851 / 1994 / 17641) |  |  |  |  |  |  |  |  |
| Wheelbase | mm |  |  |  |  | 2933 |  |  |  |  |  |  |  |  |
| Track, front/rear | mm |  |  |  |  | 1660 / 1672 |  |  |  |  |  |  |  |  |
| Turning circle | m |  |  |  |  | 12.8 |  |  |  |  |  |  |  |  |
| Tank capacity | approx ltr |  |  |  |  | 85 |  |  |  |  |  |  |  |  |
| Cooling system incl heater | ltr |  |  |  |  | 17.0 |  |  |  |  |  |  |  |  |
| Engine oil1) | ltr |  |  |  |  | 8.5 |  |  |  |  |  |  |  |  |
| Weight, unladen, to DIN/EU | kg |  |  |  |  | 2305 / 2380 |  |  |  |  |  |  |  |  |
| Max load to DIN | kg |  |  |  |  | 600 |  |  |  |  |  |  |  |  |
| Max permissible | kg |  |  |  |  | 2905 |  |  |  |  |  |  |  |  |
| Max axle load, front/rear | kg |  |  |  |  | 1400 / 1560 |  |  |  |  |  |  |  |  |
| Max trailer load (12%), braked/unbraked | kg |  |  |  |  | 3000 / 750 |  |  |  |  |  |  |  |  |
| Max roofld/max twbar downld download | kg |  |  |  |  | 100/120 |  |  |  |  |  |  |  |  |
| Luggage capacity | ltr |  |  |  |  | 620 – 1750 |  |  |  |  |  |  |  |  |
| Air resistance | CD x A |  |  |  |  | 0.38 x 2.90 |  |  |  |  |  |  |  |  |
| Power Unit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Config/No of cyls/valves |  |  |  |  |  | V90/ 8 / 4 |  |  |  |  |  |  |  |  |
| Engine technology |  |  |  |  |  | M TwinPower Turbo with exhaust gas manifold connecting both rows of cylinders, Twin Scroll Twin Turbo technology. High Precision Injection, double-VANOS |  |  |  |  |  |  |  |  |
| Capacity, effective | cc |  |  |  |  | 4395 |  |  |  |  |  |  |  |  |
| Bore / stroke | mm |  |  |  |  | 88.3 / 89.0 |  |  |  |  |  |  |  |  |
| Compression ratio | :1 |  |  |  |  | 9.3 |  |  |  |  |  |  |  |  |
| Fuel grade |  |  |  |  |  | min RON 95 |  |  |  |  |  |  |  |  |
| Max output | kW/hp |  |  |  |  | 408 / 555 |  |  |  |  |  |  |  |  |
| at | rpm |  |  |  |  | 6000 |  |  |  |  |  |  |  |  |
| Max torque | Nm/lb-ft |  |  |  |  | 680/501 |  |  |  |  |  |  |  |  |
| at | rpm |  |  |  |  | 1500 – 5650 |  |  |  |  |  |  |  |  |
| Electrical System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Battery/installation | Ah/– |  |  |  |  | 90 / luggage comp |  |  |  |  |  |  |  |  |
| Alternator | A/W |  |  |  |  | 210 / 2940 |  |  |  |  |  |  |  |  |
| Driving Dynamics and Safety |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Suspension, front |  |  |  |  |  |  | Double wishbone axle with M-specific elastokinematics; small, negative steering roll radius, anti-dive |  |  |  |  |  |  |  |
| Suspension, rear |  |  |  |  |  |  | Integral-IV rear axle with M-specific elastokinematics, multi-dimensional suspension, anti-squat and anti-dive, air suspension with automatic ride height control |  |  |  |  |  |  |  |
| Brakes, front |  |  |  |  |  |  | Four-piston swing-calliper disc brakes |  |  |  |  |  |  |  |
| Diameter |  | mm |  |  |  |  | 395 x 36, vented |  |  |  |  |  |  |  |
| Brakes, rear |  |  |  |  |  |  | Single-piston swing-calliper disc brakes |  |  |  |  |  |  |  |
| Diameter |  | mm |  |  |  |  | 385 x 24, vented |  |  |  |  |  |  |  |
| Driving stability systems |  |  |  |  |  |  | Standard: DSC incl ABS and M Dynamic Mode, CBC Cornering Brake Control, DBC Dynamic Brake Control, ADB-X electronic differential lock, Dry Braking, Fading Compensation, Start-Off Assistant; Dynamic Stability Control interacting with xDrive all-wheel drive system, Dynamic Performance Control, HDC Hill Descent Control, Adaptive Drive |  |  |  |  |  |  |  |
| Safety features |  |  |  |  |  |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, crash-activated headrests at the front, three-point inertia-reel seat belts on all seats, with belt latch tensioners, belt force limiters and belt stoppers at the front, crash sensors, rollover sensors, Tyre Defect Indicator |  |  |  |  |  |  |  |
| Steering |  |  |  |  |  |  | Rack-and-pinion steering, power-assisted, with M-specific Servotronic function |  |  |  |  |  |  |  |
| Steering transmission ratio, overall |  |  | :1 |  | 19.5 |  |  |  |  |  |  |  |  |  |
| Tyres, front/rear |  |  |  | 275/40 R20 106Y XL RSC / 315/35 R20 110Y XL RSC |  |  |  |  |  |  |  |  |  |  |
| Wheels, front/rear |  |  |  | 10J x 20 light-alloy / 11J x 20 light-alloy |  |  |  |  |  |  |  |  |  |  |

- Specifications BMW X5 M.

|  |  |  | BMW X5 M |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMW ConnectedDrive |  |  |  |  |  |  |  |  |
| Comfort |  |  |  | Optional: BMW Assist including features such as Enquiry Service, remote-control functions and V-Info+, BMW TeleServices, integration of mobile terminals |  |  |  |  |
| Infotainment |  |  |  | Optional: access to the internet, BMW Online including features such as Parkinfo, National Info, Google Directory Search, news, weather in real time, office functions, BMW Routes and music title update online |  |  |  |  |
| Safety |  |  |  | Standard: Adaptive Headlights with Bending Lights, Park Distance Control, Head-Up Display, cruise control with brake function; optional: back-up camera with Top View, High-Beam Assistant, Automatic/Enhanced Emergency Call |  |  |  |  |
| Transmission |  |  |  |  |  |  |  |  |
| Type of transmission |  |  |  | Six-speed automatic sports gearbox with Steptronic, electronic gearshift lever and shift paddles on the steering wheel |  |  |  |  |
| Gear ratios I |  | :1 |  | 4.171 |  |  |  |  |
| II |  | :1 |  | 2.340 |  |  |  |  |
| III |  | :1 |  | 1.521 |  |  |  |  |
| IV |  | :1 |  | 1.143 |  |  |  |  |
| V |  | :1 |  | 0.867 |  |  |  |  |
| VI |  | :1 |  | 0.691 |  |  |  |  |
| R |  | :1 |  | 3.403 |  |  |  |  |
| Final drive |  | :1 |  | 3.909 |  |  |  |  |
| Performance |  |  |  |  |  |  |  |  |
| Power-to-weight ratio |  | kg/kW |  | 5.6 |  |  |  |  |
| Output per litre |  | kW/hp |  | 92.8/126.2 |  |  |  |  |
| Acceleration 0–100 km/h |  | sec |  | 4.7 |  |  |  |  |
| 0–200 km/h |  | sec |  | 16.9 |  |  |  |  |
| Standing-start km |  | sec |  | 23.5 |  |  |  |  |
| In fourth/fifth gear 80–120 km/h |  | sec |  | 4.3 / 5.1 |  |  |  |  |
| Top speed |  | km/h |  | 250 (275)3) |  |  |  |  |
| Offroad Features |  |  |  |  |  |  |  |  |
| Overhang angle, front/rear |  | ° |  | 22 / 21.5 |  |  |  |  |
| Ramp angle |  | ° |  | 18.6 |  |  |  |  |
| Ground clearance, unladen |  | mm |  | 207 |  |  |  |  |
| BMW EfficientDynamics |  |  |  |  |  |  |  |  |
| BMW EfficientDynamics, standard features |  |  |  | Brake Energy Regeneration, on-demand control of ancillary units, intelligent lightweight construction, detachable a/c compressor, anti-roll stability control with on-demand pump action |  |  |  |  |
| Fuel Cons in the EU Cycle |  |  |  |  |  |  |  |  |
| Urban | ltr/100km |  |  | 19.3 |  |  |  |  |
| Extra-urban | ltr/100km |  |  | 10.8 |  |  |  |  |
| Combined | ltr/100km |  |  | 13.9 |  |  |  |  |
| CO2 | g/km |  |  | 325 |  |  |  |  |
| Emission rating |  |  |  | EU5 |  |  |  |  |

Specifications apply to ACEA markets/homologation-relevant data apply in part only to Germany (weight)

1)  Height without roof aerial: 1,726 mm

2)  Oil change

3)  In conjunction with optional M Driver’s Package