- Specifications BMW 7 Series Saloon.
730d, 740d

|  |  |  | BMW 730d |  |  |  | BMW 740d |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |  |  |
| No of doors/seats |  |  | 4 / 5 |  |  |  | 4 / 5 |
| Length/width/height (unladen) | mm |  | 5072 / 1902 / 1479 |  |  |  | 5072 / 1902 / 1479 |
| Wheelbase | mm |  | 3070 |  |  |  | 3070 |
| Track, front/rear | mm |  | 1611 / 1650 |  |  |  | 1611 / 1650 |
| Ground clearance | mm |  | 141 |  |  |  | 148 |
| Turning circle | m |  | 12,2 |  |  |  | 12,2 |
| Tank capacity | app ltr |  | 80 |  |  |  | 80 |
| Cooling system incl heater | ltr |  | 8.9 |  |  |  | 9.0 |
| Engine oil1) | ltr |  | 7.2 |  |  |  | 7.2 |
| Weight, unladen, to DIN/EU | kg |  | 1865 / 1940 |  |  |  | 1875 / 1950 |
| Max load to DIN | kg |  | 640 |  |  |  | 630 |
| Max permissible | kg |  | 2505 |  |  |  | 2505 |
| Max axle load, front/rear | kg |  | 1210 / 1355 |  |  |  | 1200 / 1375 |
| Max trailer load (12%), braked/unbraked | kg |  | 2100 / 750 |  |  |  | 2100 / 750 |
| Max roofload/max towbar downl | kg |  | 100 / 100 |  |  |  | 100 / 100 |
| Luggage comp capacity | ltr |  | 500 |  |  |  | 500 |
| Air drag | Cd x A |  | 0.29 x 2.41 |  |  |  | 0.30 x 2.41 |
| Power Unit |  |  |  |  |  |  |  |
| Power Unit |  |  |  |  |  |  |  |
| Config/No of cyls/valves |  |  | Straight-six / 4 |  |  |  | Straight-six / 4 |
| Engine technology |  |  | Turbocharger with variable intake geometry, common-rail direct fuel injection with piezo-injectors (max injection pressure: 1800 bar) |  |  | BMW TwinPower Turbo (Variable Twin Turbo), common-rail direct fuel injection with piezo-injectors (max injection pressure: 2000 bar) |  |
| Capacity, effective | cc |  | 2993 |  |  |  | 2993 |
| Stroke / bore | mm |  | 90.0 / 84.0 |  |  |  | 90.0 / 84.0 |
| Compression ratio | :1 |  | 16.5 |  |  |  | 16.5 |
| Fuel |  |  | Diesel |  |  |  | Diesel |
| Max output | kW/hp |  | 180 / 245 |  |  |  | 225 / 306 |
| at | rpm |  | 4 000 |  |  |  | 4400 |
| Max torque | Nm/lb-ft |  | 540/398 |  |  |  | 600/442 |
| at | rpm |  | 1750–3 000 |  |  |  | 1500-2500 |
| Electrical System |  |  |  |  |  |  |  |
| Battery/location | Ah/– |  | 90 / luggage comp |  |  |  | 90 / luggage comp |
| Alternator | A/W |  | 180 / 2 520 |  |  |  | 180 / 2520 |
| Driving Dynamics and Safety |  |  |  |  |  |  |  |
| Suspension, front |  | Double track control arm axle with separate lower track arm level in aluminium, small steering roll radius, anti-dive |  |  |  |  |  |
| Suspension, rear |  | Integral-V multi-arm axle in aluminium with steering function, anti-squat and anti-dive, double acoustic separation (air suspension with self-levelling standard on long-wheelbase version) |  |  |  |  |  |
| Brakes, front |  | Single-piston swing-calliper disc brakes |  |  |  |  |  |
| Diameter | mm | 348 x 36 / vented |  |  |  |  | 348 x 36 / vented |
| Brakes, rear |  | Single-piston swing-calliper disc brakes |  |  |  |  |  |
| Diameter | mm | 345 x 24 / vented |  |  |  |  | 345 x 24 / vented |
| Driving stability systems |  | Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant, Dynamic Damping Control, optional: Dynamic Drive anti-roll stability management |  |  |  |  |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, active headrests at the front, three-point inertia-reel seat belts on all seats, belt latch tensioners at the front, crash sensors, Tyre Defect Indicator |  |  |  |  |  |
| Steering |  | Rack-and-pinion power steering with speed-related steering assistance (Servotronic) and CO2-optimised pump; optional: Integral Active Steering |  |  |  |  |  |
| Steering transmission, overall | :1 | 19.1 |  |  |  |  | 19.1 |
| Tyres, front/rear |  | 245/55 R17 102W RSC |  |  |  |  | 245/50 R18 100Y RSC |
| Rims, front/rear |  | 8J x 17 light-alloy |  |  |  |  | 8J x 18 light-alloy |

- Specifications BMW 7 Series Saloon,

|  |  |  | BMW 730d |  |  |  |  | BMW 740d |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMW ConnectedDrive |  |  |  |  |  |  |  |  |  |
| Comfort |  |  |  |  | Optional: BMW Assist including Enquiry Service, remote-control function and V-Info+, BMW TeleServices, integration of mobile terminals |  |  |  |  |
| Infotainment |  |  |  | Optional: access to the internet, BMW Online including features such as Parkinfo, Nationalinfo, Google Directory Search, News, Realtime Weather, BMW Routes, Office Functions, Bluetooth Audio Streaming, online music title update |  |  |  |  |  |
| Safety |  |  |  | Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant, Park Distance Control, back-up camera, Surround View with Side View and Top View, BMW Night Vision with detection of persons, Head-Up Display, Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Automatic/Enhanced Emergency Call |  |  |  |  |  |
| Transmission |  |  |  |  |  |  |  |  |  |
| Type of transmission |  |  |  |  | Six-speed automatic transmission with Steptronic function |  |  |  |  |
| Gear ratios I |  | :1 |  |  | 4.171 |  |  |  | 4.171 |
| II |  | :1 |  |  | 2.340 |  |  |  | 2.340 |
| III |  | :1 |  |  | 1.521 |  |  |  | 1.521 |
| IV |  | :1 |  |  | 1.143 |  |  |  | 1.143 |
| V |  | :1 |  |  | 0.867 |  |  |  | 0.867 |
| VI |  | :1 |  |  | 0.691 |  |  |  | 0.691 |
| Reverse |  | :1 |  |  | 3.403 |  |  |  | 3.403 |
| Final drive |  | :1 |  |  | 2.813 |  |  |  | 3.077 |
| Performance |  |  |  |  |  |  |  |  |  |
| Power-to-weight ratio | kg/kW |  | 10.4 |  |  |  |  | 8.3 |  |
| Output per litre | kW/hp |  | 60.1/81.7 |  |  |  |  | 75.2/102.3 |  |
| Acceleration 0–100 km/h | sec |  | 7.2 |  |  |  |  | 6.3 |  |
| Standing-start km | sec |  | 27.3 |  |  |  |  | 26.0 |  |
| Top speed | km/h |  | 245 |  |  |  |  | 250 |  |
| BMW EfficientDynamics |  |  |  |  |  |  |  |  |  |
| BMW EfficientDynamics standard features |  |  | Brake Energy Regeneration, Varioserv steering assistance pump, intelligent lightweight technology, air flap control, on-demand fuel pump, on-demand coolant pump, final drive with optimised thermal management, tyres with reduced roll resistance, a/c compressor detachable, optional Dynamic Drive with on-demand pump control |  |  |  |  |  |  |
| Fuel Consump in EU Cycle |  |  |  |  |  |  |  |  |  |
| Urban | ltr/100 km |  | 9.0 |  |  |  |  | 9.0 |  |
| Extra-urban | ltr/100 km |  | 5.5 |  |  |  |  | 5.7 |  |
| Combined | ltr/100 km |  | 6.8 |  |  |  |  | 6.9 |  |
| CO2 | g/km |  | 178 |  |  |  |  | 181 |  |
| Emission rating |  |  | EU5 |  |  |  |  | EU5 |  |

Specifications applicable to ACEA markets/homologation data applicable in part only to Germany (weight),

- Height with roof fin: 1488 mm

- Oil change

- EU6 with optional BluePerformance technology