- Specifications BMW 7 Series Saloon.
740i, 750i, 760i

|  |  | BMW 740i |  | BMW 750i | BMW 760i |  |
| --- | --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |  |
| No of doors/seats |  | 4 / 5 |  | 4 / 5 | 4 / 5 |  |
| Length/width/height (unladen) | mm | 5072 / 1902 / 1479 |  | 5072 / 1902 / 1479 | 5072 / 1902 / 1473 |  |
| Wheelbase | mm | 3070 |  | 3070 | 30370 |  |
| Track, front/rear | mm | 1611 / 1650 |  | 1611 / 1650 | 1611 / 1650 |  |
| Ground clearance | mm | 144 |  | 144 | 149 |  |
| Turning circle | m | 12.2 |  | 12.2 | 12.2 |  |
| Tank capacity | app ltr | 82 |  | 82 | 82 |  |
| Cooling system incl heater | ltr | 9.0 |  | 13.5 | 15.1 |  |
| Engine oil1) | ltr | 6.5 |  | 8.5 | 10.5 |  |
| Weight, unladen, to DIN/EU | kg | 1860 (1935) |  | 1945 (2020) | 2105 (2180) |  |
| Max load to DIN | kg | 645 |  | 630 | 590 |  |
| Max permissible | kg | 2505 |  | 2575 | 2695 |  |
| Max axle load, front/rear | kg | 1200 / 1365 |  | 1270 / 1365 | 1365 / 1440 |  |
| Max trailer load (12%), braked/unbraked | kg | 2100 / 750 |  | 2100 / 750 | 2100 / 750 |  |
| Max roofload/max towbar downl | kg | 100 / 100 |  | 100 / 100 | 100 / 100 |  |
| Luggage comp capacity | ltr | 500 |  | 500 | 500 |  |
| Air drag | Cd x A | 0.30 x 2.41 |  | 0.31 x 2.41 | 0.31 x 2.41 |  |
| Power Unit |  |  |  |  |  |  |
| Power Unit |  |  |  |  |  |  |
| Config/No of cyls/valves |  | Straight-six / 4 |  | V8 / 4 | V12 / 4 |  |
| Engine technology |  | BMW Twin Turbo, second-generation direct injection (High Precision Injection) | BMW TwinPower Turbo, second-generation direct injection (High Precision Injection) |  |  | BMW TwinPower Turbo, second-generation direct injection (High Precision Injection) |
| Capacity, effective | cc | 2979 |  | 4395 | 5972 |  |
| Stroke / bore | mm | 89.6 / 84.0 |  | 88.3 / 89.0 | 80.0 / 89.0 |  |
| Compression ratio | :1 | 10.2 |  | 10.0 | 10.0 |  |
| Fuel |  | min RON 95 |  | min RON 91 | min RON 91 |  |
| Max output | kW/hp | 240 / 326 |  | 300 / 407 | 400 / 544 |  |
| at | rpm | 5 800 |  | 5 500–6 400 | 5250 |  |
| Max torque | Nm/lb-ft | 450/332 |  | 600/442 | 750/553 |  |
| at | rpm | 1500–4 500 |  | 1 750–4 500 | 1500–5000 |  |
| Electrical System |  |  |  |  |  |  |
| Battery/location | Ah/– | 90 / luggage comp |  | 90 / luggage comp | 90 / luggage comp |  |
| Alternator | A/W | 220 / 3080 |  | 220 / 3080 | 210 / 2940 |  |
| Driving Dynamics and Safety |  |  |  |  |  |  |
| Suspension, front |  | Double track control arm axle with separate lower track arm level in aluminium construction, small steering roll radius, anti-dive |  |  |  |  |
| Suspension, rear |  | Integral-V multi-arm axle in alu with steering function, anti-squat and anti-dive, double acoustic separation, air suspension with automatic self-levelling (BMW 760i) (air suspension with self-levelling standard on long-wheelbase version) |  |  |  |  |
| Brakes, front |  | Single-piston swing-calliper disc brakes |  |  |  |  |
| Diameter | mm | 374 x 36 / vented |  | 374 x 36 / vented | 374 x 36 / vented |  |
| Brakes, rear |  | Single-piston fixed-calliper disc brakes |  |  |  |  |
| Diameter | mm | 345 x 24 / vented |  | 370 x 24 / vented | 370 x 24 / vented |  |
| Driving stability systems |  | Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant, Dynamic Damping Control, optional: Dynamic Drive anti-roll stability management (standard in the BMW 760i) |  |  |  |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, active headrests at the front, three-point inertia-reel seat belts on all seats, belt latch tensioners at the front, crash sensors, Tyre Defect Indicator |  |  |  |  |
| Steering |  | Rack-and-pinion power steering with speed-related steering assistance (Servotronic) and CO2-optimised pump; optional: Integral Active Steering |  |  |  |  |
| Steering transmission, overall | :1 | 19.1 |  | 19.1 | 19.1 |  |
| Tyres, front/rear |  | 245/50 R18 100Y RSC |  | 245/50 R18 100Y RSC | 245/50 R18 100Y RSC |  |
| Rims, front/rear |  | 8J x 18 light-alloy |  | 8J x 18 light-alloy | 8J x 18 light-alloy |  |

- Specifications BMW 7 Series Saloon.

|  |  |  |  | BMW 740i |  |  | BMW 750i |  | BMW 760i |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMW ConnectedDrive |  |  |  |  |  |  |  |  |  |
| Comfort |  |  |  |  |  | Optional: BMW Assist including Enquiry Service, remote-control function and V-Info+, BMW TeleServices, integration of mobile terminals |  |  |  |
| Infotainment |  |  |  |  | Optional: access to the internet, BMW Online including features such as Parkinfo, Nationalinfo, Google Directory Search, News, Realtime Weather, BMW Routes, Office Functions, Bluetooth Audio Streaming, online music title update |  |  |  |  |
| Safety |  |  |  |  | Optional: Adaptive Headlights with Bending Lights (standard in the BMW 760i), High-Beam Assistant, Park Distance Control (standard in the BMW 760i), Side View, Top View, back-up camera (standard in the BMW 760i), BMW Night Vision with detection of persons, Head-Up Display, Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Automatic/Enhanced Emergency Call |  |  |  |  |
| Transmission |  |  |  |  |  |  |  |  |  |
| Type of transmission |  |  | Six-speed automatic transmission with Steptronic |  |  |  |  |  | Eight-speed automatic transmission with Steptronic |
| Gear ratios I |  | :1 | 4.171 |  |  |  | 4.171 |  | 4.714 |
| II |  | :1 | 2.340 |  |  |  | 2.340 |  | 3.143 |
| III |  | :1 | 1.521 |  |  |  | 1.521 |  | 2.106 |
| IV |  | :1 | 1.143 |  |  |  | 1.143 |  | 1.667 |
| V |  | :1 | 0.867 |  |  |  | 0.867 |  | 1.285 |
| VI |  | :1 | 0.691 |  |  |  | 0.691 |  | 1.000 |
| VII |  | :1 |  |  |  |  |  |  | 0.839 |
| VIII |  | :1 |  |  |  |  |  |  | 0.667 |
| Reverse |  | :1 | 3.403 |  |  |  | 3.403 |  | 3.317 |
| Final drive |  | :1 | 3.462 |  |  |  | 3.462 |  | 2.813 |
| Performance |  |  |  |  |  |  |  |  |  |
| Power-to-weight ratio | kg/kW |  |  | 7.8 |  |  | 6.5 | 5.3 |  |
| Output per litre | kW/hp |  |  | 80.6/109.6 |  |  | 68.3/92.9 | 67.0/91.1 |  |
| Acceleration 0–100 km/h | sec |  |  | 5.9 |  |  | 5.2 | 4.6 |  |
| Standing-start km | sec |  |  | 25.5 |  |  | 24.1 | 22.5 |  |
| Top speed | km/h |  |  | 250 |  |  | 250 | 250 |  |
| BMW EfficientDynamics |  |  |  |  |  |  |  |  |  |
| BMW EfficientDynamics standard features |  |  |  | Brake Energy Regeneration, Varioserv steering assistance pump, intelligent lightweight technology, air flap control (BMW 740i), on-demand fuel pump, on-demand coolant pump, final drive with optimised thermal management, tyres with reduced roll resistance, a/c compressor detachable, optional Dynamic Drive with on-demand pump control (standard in the BMW 760i) |  |  |  |  |  |
| Fuel Consump in EU Cycle |  |  |  |  |  |  |  |  |  |
| Urban | ltr/100 km |  |  | 13.8 |  |  | 16.4 | 18.8 |  |
| Extra-urban | ltr/100 km |  |  | 7.6 |  |  | 8.5 | 9.5 |  |
| Combined | ltr/100 km |  |  | 9.9 |  |  | 11.4 | 12.9 |  |
| CO2 | g/km |  |  | 232 |  |  | 266 | 299 |  |
| Emission rating |  |  |  | EU5 |  |  | EU5 | EU5 |  |

Specifications applicable to ACEA markets/homologation data applicable in part only to Germany (weight).

1) Height with roof fin: 1488 mm (BMW 740i, BMW 750i), 1479 mm (BMW 760i)

2)  Oil change