- Specifications BMW 7 Series Saloon.
750i xDrive

|  |  | BMW 750i xDrive |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |
| No of doors/seats |  | 4 / 5 |  |  |  |
| Length/width/height (unladen) | mm | 5072 / 1902 / 1479 |  |  |  |
| Wheelbase | mm | 3070 |  |  |  |
| Track, front/rear | mm | 1611 / 1650 |  |  |  |
| Ground clearance | mm | 148 |  |  |  |
| Turning circle | m | 12.5 |  |  |  |
| Tank capacity | app ltr | 82 |  |  |  |
| Cooling system incl heater | ltr | 13.5 |  |  |  |
| Engine oil1) | ltr | 8.5 |  |  |  |
| Weight, unladen, to DIN/EU | kg | 2030 / 2105 |  |  |  |
| Max load to DIN | kg | 630 |  |  |  |
| Max permissible | kg | 2660 |  |  |  |
| Max axle load, front/rear | kg | 1335 / 1415 |  |  |  |
| Max trailer load (12%), braked/unbraked | kg | 2100 / 750 |  |  |  |
| Max roofload/max towbar downl | kg | 100 / 100 |  |  |  |
| Luggage comp capacity | ltr | 500 |  |  |  |
| Air drag | Cd x A | 0.31 x 2.41 |  |  |  |
| Power Unit |  |  |  |  |  |
| Config/No of cyls/valves |  | V8 / 4 |  |  |  |
| Engine technology |  | BMW TwinPower Turbo, second-generation direct fuel injection (High Precision Injection) |  |  |  |
| Capacity, effective | cc | 4395 |  |  |  |
| Stroke / bore | mm | 88.3 / 89.0 |  |  |  |
| Compression ratio | :1 | 10.0 |  |  |  |
| Fuel |  | min RON 91 |  |  |  |
| Max output | kW/hp | 300 / 407 |  |  |  |
| at | rpm | 5500–6400 |  |  |  |
| Max torque | Nm/lb-ft | 60o/442 |  |  |  |
| at | rpm | 1750–4500 |  |  |  |
| Electrical System |  |  |  |  |  |
| Battery/location | Ah/– | 90 / luggage comp |  |  |  |
| Alternator | A/W | 220 / 3080 |  |  |  |
| Driving Dynamics and Safety |  |  |  |  |  |
| Suspension, front |  | Double track control arm axle with separate lower track arm level in aluminium construction, small steering roll radius, anti-dive |  |  |  |
| Suspension, rear |  | Integral-V multi-arm axle in aluminium with steering function, anti-squat and anti-dive, double acoustic separation (air suspension with self-levelling standard on long-wheelbase version) |  |  |  |
| Brakes, front |  | Single-piston swing-calliper disc brakes |  |  |  |
| Diameter | mm | 374 x 36 / vented |  |  |  |
| Brakes, rear |  | Single-piston swing-calliper disc brakes |  |  |  |
| Diameter | mm | 370 x 24 / vented |  |  |  |
| Driving stability systems |  | Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant, networked with xDrive in ICM Integrated Chassis Management, Dynamic Damper Control, optional: Dynamic Drive anti-roll stability management |  |  |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, active headrests at the front, three-point inertia-reel seat belts on all seats, belt latch tensioners at the front, crash sensors, Tyre Defect Indicator |  |  |  |
| Steering |  | Rack-and-pinion power steering with speed-related steering assistance (Servotronic) and CO2-optimised pump |  |  |  |
| Steering transmission, overall | :1 | 19.1 |  |  |  |
| Tyres, front/rear |  | 245/50 R18 100Y RSC |  |  |  |
| Rims, front/rear |  | 8J x 18 light-alloy |  |  |  |

- Specifications BMW 7 Series Saloon.

|  |  |  | BMW 750i xDrive |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMW ConnectedDrive |  |  |  |  |  |  |  |  |  |
| Comfort |  |  |  |  | Optional: BMW Assist including Enquiry Service, remote-control function and V-Info+, BMW TeleServices, integration of mobile terminals |  |  |  |  |
| Infotainment |  |  |  | Optional: access to the internet, BMW Online including features such as Parkinfo, Nationalinfo, Google Directory Search, News, Realtime Weather, Office Functions, Bluetooth Audio Streaming and online music title update |  |  |  |  |  |
| Safety |  |  |  | Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant, Park Distance Control, back-up camera, Surround View incl Side View and Top View, BMW Night Vision with detection of persons, Head-Up Display, Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Automatic/Enhanced Emergency Call |  |  |  |  |  |
| Transmission |  |  |  |  |  |  |  |  |  |
| Type of transmission |  |  |  |  | Six-speed automatic transmission with Steptronic function |  |  |  |  |
| Gear ratios I |  | :1 |  |  | 4.171 |  |  |  |  |
| II |  | :1 |  |  | 2.340 |  |  |  |  |
| III |  | :1 |  |  | 1.521 |  |  |  |  |
| IV |  | :1 |  |  | 1.143 |  |  |  |  |
| V |  | :1 |  |  | 0.867 |  |  |  |  |
| VI |  | :1 |  |  | 0.691 |  |  |  |  |
| Reverse |  | :1 |  |  | 3.403 |  |  |  |  |
| Final drive |  | :1 |  |  | 3.462 |  |  |  |  |
| Performance |  |  |  |  |  |  |  |  |  |
| Power-to-weight ratio | kg/kW |  | 6.8 |  |  |  |  |  |  |
| Output per litre | kW/hp |  | 68.3/92.9 |  |  |  |  |  |  |
| Acceleration 0–100 km/h | sec |  | 5.1 |  |  |  |  |  |  |
| Standing-start km | sec |  | 24.0 |  |  |  |  |  |  |
| Top speed | km/h |  | 250 |  |  |  |  |  |  |
| BMW EfficientDynamics |  |  |  |  |  |  |  |  |  |
| BMW EfficientDynamics standard features |  |  | Brake Energy Regeneration, Varioserv steering assistance pump, intelligent lightweight technology, air flap control, on-demand fuel pump, on-demand coolant pump, final drive with optimised thermal management, tyres with reduced roll resistance, a/c compressor detachable, Dynamic Drive with on-demand pump control |  |  |  |  |  |  |
| Fuel Consump in EU Cycle |  |  |  |  |  |  |  |  |  |
| Urban | ltr/100 km |  | 17.1 |  |  |  |  |  |  |
| Extra-urban | ltr/100 km |  | 8.9 |  |  |  |  |  |  |
| Combined | ltr/100 km |  | 11.9 |  |  |  |  |  |  |
| CO2 | g/km |  | 278 |  |  |  |  |  |  |
| Emission rating |  |  | EU5 |  |  |  |  |  |  |

Specifications applicable to ACEA markets/homologation data applicable in part only to Germany (weight).

1) Height with roof fin: 1484 mm

2)  Oil change