–4–

IMMEDIATE

David J. Buchko

BMW Product Communications Manager

201-307-3789 / dave.buchko@bmwna.com

Gordon B. Keil

BMW Product Communications Specialist

201-307-3790 / gordon.keil@bmwna.com

2004 BMW 7 Series 

Specifications

General
745i
745Li
760i
760Li

Curb weight, lb.
4376
4464
4782
4872

GVWR, lb.
5434
5523
5820
5930

Weight distribution,


front/rear, %
50.4/49.6
50.6/49.4
51.6/48.4
51.6/48.4

Wheelbase, in.
117.7
123.2
117.7
123.2

Track, front/rear, in.
62.1/62.3 
  

Length, in.
198.0
203.5
198.0
203.5
Width, in.
74.9 1


Height, in.
58.7 1
Accommodations

Seating capacity, persons
5 2
Shoulder room, front/rear, in.
59.3/58.7
59.3/58.9
59.3/58.7
59.3/58.9
Head room, front/rear, in.
39.2/38.5 1
Leg room, front/rear
41.3/37.2
41.3/43.4
41.3/37.2
41.3/43.4
EPA interior volume, cu ft.
104.0
111.0
104.0
111.0
EPA cargo volume, cu ft.
18.0
18.0
18.0
18.0 

Body
745i
745Li
760i
760Li

Type
4-door sedan 1

Aerodynamic drag coefficient


(CD)
0.29 1
EPA size classification
Large 1

Engine & electrical

Engine type

DOHC 32-valve (4-cam)
DOHC 48-valve (4-cam)




V-8 


V-12 

Engine features
Valvetronic variable valve lift & Double VANOS 



variable valve timing 1


Steplessly variable
Direct fuel injection 5



intake system 4
Cylinder block & heads
Aluminum 1

Bore x stroke, mm/in.
92.0 x 82.7/

89.0 x 80.0/



3.62 x 3.26 4

3.50 x 3.15 5

Displacement, cc/cu in.
4398/268 4

5972/364 5

Compression ratio
10.5:1 4

11.3:15

Power @ rpm, hp
325 @ 6100 4

438 @ 6000 5

Torque @ rpm, lb-ft.
330 @ 3600 4

444 @ 3950 5

Engine-management system
Motronic ME9 4
Motronic MED9 5

Fuel requirement
Premium unleaded 2

Fuel capacity, U.S. gal.
23.3 1

Battery capacity, amp-hr.
110 1

Alternator output, amp./W
180/2520 1

Drivetrain
745i
745Li
760i
760Li

Drive system
Front engine/rear drive 1

Automatic transmission
ZF 6 HP 26, 6-speed 1

Ratios:
1st
4.17:1 1



2nd
2.34:11



3rd
1.52:1 1


4th
1.14:1 1


5th
0.87:1 1


6th
0.69:1 1


Reverse
3.40:1 1
Final drive ratio
3.38:1 4

3.15:15
Chassis

Body/frame construction
Unitized all-steel structure with aluminum



hood & front fenders 1

Front suspension
Strut-type in aluminum; double-pivot lower arms,



coil springs, twin-tube gas-pressure shock



absorbers, Active Roll Stabilization, aluminum



subframe (745i/Li with Adaptive Ride Package,



760i/Li std.: Electronic Damping Control; 745i/Li



with Sport Package: sport suspension calibration)

Rear suspension
4-link Integral Link suspension in aluminum, coil



springs, twin-tube gas-pressure shock absorbers,



Active Roll Stabilization, aluminum subframe



(745i/Li with Adaptive Ride Package, 760i/Li std.:



self-leveling air springs & Electronic Damping



Control; 745i/Li with Sport Package: sport



suspension calibration)
Steering type
Rack & pinion, vehicle-speed-sensitive power assist



& variable ratio 1


Turns lock-to-lock
3.07 1

Turning circle, ft.
39.8 4
41.3 5
39.8 4
41.3 5
Chassis (cont.)
745i
745Li
760i
760Li

4-wheel ventilated disc brakes:


Diameter x thickness,
348 x 30 / 13.7 x 1.18 front/

mm/in.
345 x 24 / 13.6 x 0.94 rear 1


Assist
Vacuum 1

Wheels: 
standard, cast alloy
18 x 8.0 4

20 x 9.0 F/
19 x 9.0 F/






20 x 10.0 R
19 x 10.0 R



optional
Cast alloy, 19 x 9.0 F /
NA
Forged




19 x 10.0 R 4


alloy,







18 x 8.0


Tires:
standard
245/50R-18 V-rated
760i: 245/40R-20 F/




all-season 4

275/35R-20 R






Y-rated performance






760Li: 245/45R-19 F/






275/40R-19 R






W-rated performance

Tires:
optional
245/45R-19 front /
760Li only: 245/50R-18




275/40R-19 rear
V-rated all-season




W-rated performance 4

Stability-enhancement system
Dynamic Stability Control (DSC), including Dynamic




Traction Control, antilock braking (ABS), Dynamic Brake




Control & cornering/braking stability enhancement 1
Performance data


Acceleration, 0-60 mph, sec.  

5.9 4

5.4 5

Top speed, mph
150 (electronically limited) 1
Fuel economy, EPA est. MPG,


city/highway
18/26 4

15/23 5

F – Front     R – Rear

� – Specification applies to all models.


� – With 18-in. wheels (not available on 760i).


� – Optional rear climate control & coolbox reduce cargo volume.


� – Specification applies to 745i & 760i.


� – Specification applies to 745Li & 760Li.


� – VANOS = VAriable NOckenwellen Steuerung = variable camshaft control, or variable valve timing.


� – BMW AG test results. Actual acceleration results may vary depending on specification of vehicle; road and environmental conditions; testing procedures and driving style. These results should be used for comparison only, and verification should not be attempted on public roads. BMW urges you to obey all posted speed limits and to please wear your safety belt at all times.





–more–
–more–


